# NA-16 Architecture

Volume 1: Programmers Reference

Document Revision: 0.1
Architecture Revision: 1.0
Date: 6th of June

# CONTENTS

PAGE

Chapter 1
Introduction
1.1 	Purpose of this Manual...........................................X
1.2 	Architectural Overview...........................................X
1.3 	Terminology......................................................X
1.4 	Revision History.................................................X

Chapter 2
Programmer’s Model
2.1 	Register Set
2.2 	General Purpose Registers
2.3 	Special Purpose Registers
2.4 	Program Counter
2.5 	Status Register
2.6 	Privilege levels

Chapter 3
Data Types and Memory Model
3.1 	Byte and Word Types
3.2 	Endianness
3.3 	Address Space
3.4 	Alignment Rules
3.5 	Memory Accesses

Chapter 4
Instruction Encoding
4.1 	Base Instruction Format
4.2 	Instruction Classes
4.3 	Opcode Encoding
4.4 	Prefix Encoding
4.5 	Instruction Lengths

Chapter 5
Addressing Modes
5.1 	Register
5.2 	Immediate
5.3 	Register Indirect
5.4 	Absolute
5.5 	Register Indirect + Displacement

CONTENTS

PAGE

5.6 	SIB
5.7 	SIB + Displacement

Chapter 6
Operand Sizes
6.1 	Default Operand Size
6.2 	AEX Prefix
6.3 	Immediate Sizes
6.4 	Memory Access Sizes
6.5 	Displacement Sizes

Chapter 7
Instruction Set Reference
7.1 	Arithmetic Instructions
7.2 	Logical Instructions
7.3 	Data Movement Instructions
7.4 	Control Flow Instructions
7.5 	System Instructions

Chapter 8
Flags and Condition Codes
8.1 	Zero Flag
8.2 	Carry Flag
8.3 	Overflow Flag
8.4 	Sign Flag
8.5	Interrupt Flag
8.6 	Condition Evaluation

Chapter 9
Programming Example
9.1 	Function Call Example
9.2 	Loop Example
9.3 	Memory Copy Example
9.4 	Stack Usage Example

Appendix A
Opcode Summary

CONTENTS

PAGE

Appendix B
Prefix Summary

Appendix C
Register Summary

Appendix D
Instruction Quick Reference

# CHAPTER 1
INTRODUCTION

## PURPOSE OF THIS MANUAL

This manual provides information for programmers, compiler developers, and assembler developers who target the NA-16 architecture. It serves as the primary reference for the architectural features visible to software, including the programmer's model, instruction set, memory model, and execution environment.

This manual describes the behaviour of the architecture as observed by software and does not generally cover implementation-specific details of individual processor designs.

## ARCHITECTUAL OVERVIEW

The processor implements a 16-bit instruction set architecture. Instructions are encoded using a 16-bit base instruction word and may be preceded by one or more prefix words that modify operand interpretation, addressing modes, or operand sizes.

The instruction set is organised into four major classes:

| Class | Description |
| --- | --- |
| 0 | Arithmetic and Logic operations |
| 1 | Data movement operations |
| 2 | Control flow operations |
| 3 | System operations |

The architecture supports register, immediate, and memory operands. Additional addressing modes, including indexed addressing, are provided through the use of operand extension prefixes.

## TERMINOLOGY

The following terminology is used throughout this manual:

Byte
: An 8-bit quantity.

Word
: A 16-bit quantity.

Operand
: A value consumed or produced by an instruction.

Destination Operand
: The operand receiving the result of an operation.

Source Operand
: An operand providing input to an operation.

Prefix
: An instruction word or byte that modifies the interpretation of the following instruction.

Addressing Mode
: The mechanism used to obtain an operand value.

Instruction
: A machine operation encoded for execution by the processor.

Register
: A storage location within the processor that may be used to hold data, addresses, or processor state.

## 1.4 	REVISION HISTORY

This section is dedicated to the documentation of the changes to both the ISA and architecture and Documentation Volume

### 1.4.1 	ISA REVISION HISTORY

### 1.4.2	VOLUME REVISION HISTORY

# CHAPTER 2
PROGRAMMER’S MODEL

## 2.1 	REGISTER SET

The processor provides a set of general-purpose and special-purpose registers accessible to software.

| Register | Type | Description |
| --- | --- | --- |
| R0-R7 | General Purpose | General purpose registers |
| PC | Special Purpose | Pointer to the next instruction |
| SP | Special Purpose | Pointer to the top of the stack |
| BP | Special Purpose | Pointer to the start of a stack frame |
| FR | Special Purpose | Holds status flags |
| CR0 | Special Purpose | Kernel page table pointer |
| CR1 | Special Purpose | User page table pointer |

CR0 and CR1 have further detailing in volume [insert volume here]

## 2.2		GENERAL PURPOSE REGISTERS

Out of the register set the processor defines general-purpose registers to which can be used for anything.

## 2.3 	SPECIAL PURPOSE REGISTERS

The other set of registers in the register set are special-purpose registers. These registers were implemented with a specific use case in mind.

## 2.4 	PROGRAM COUNTER

The Program Counter (PC) contains the address of the next instruction to be executed.

Control-flow instructions modify the value of the Program Counter in order to alter execution flow.

## 2.5		Status Register

The status register – or flags register - contains condition flags that reflect the results of arithmetic and logical operations. It also contains other control values unrelated to arithmetic and logical operations further detailed in volume [insert volume number here].

The following flags are defined:

| Flag | Description |
| --- | --- |
| ZF | Set when the result of an operation is zero |
| CF | Set when an arithmetic operation generates a carry or borrow |
| OF | Set when a signed arithmetic overflow occurs. |
| SF | Reflects the sign of the result. |
| IEF | Enables maskable interrupts |
| DF | Enables Debug Mode |
| VMEF | Enables virtual memory mode |
| UF | Sets CPU to user mode |

## 2.6 PRIVILEGE LEVELS

The NA-16 architecture has 2 distinct privilege levels: Kernel or Privileged and User.

In kernel mode the CPU can perform all instructions available in the instruction set. There are no restrictions and this mode is extremely dangerous. All registers are accessible and manipulable.

In user mode only a specific subset of instructions is usable (See Chapter 4 for more details). Only the general-purpose registers, SP and BP are directly manipulable. Direct access to any other register will trigger an NMI (Non Maskable Interrupt) further detailed in volume [Insert Volume number here]. When the VMEF is set to 1 memory access is also limited by the access flags of page table entries. Virtual memory is explained further in volume [insert volume here].

# CHAPTER 3
DATA TYPES AND MEMORY MODEL

## 3.1 	BYTE AND WORD TYPES

The NA-16 architecture works with 2 main types of data. A byte and a Word. A byte is a set of 8 bits and is the smallest unit that can easily be manipulated. A word is the native size of the architecture. A word is a set of 16 bits and is the largest unit that can be manipulated.

## 3.2 	ENDIANNESS

Endianness defines the in which bytes of a multi-byte value are stored in memory. This architecture uses little-endian format.

In a little-endian format, the least significant byte (LSB) of a value is stored at the lowest memory address, while the most significant byte (MSB) is stored at the highest memory address.

For example, a word with the value of 0x1234 stored at 0x1000 would be represented as:

| Address | Value |
| --- | --- |
| 0x1000 | 0x34 |
| 0x1001 | 0x12 |

## 3.3 	ADDRESS SPACE

The NA-16 architecture features a 16-bit native address space, while physically supporting 32 address lines. By default, the CPU operates within a 64 KB address space. [Something about the MMU and virtual memory maybe idk]

## 3.4 	ALIGNMENT RULES

[not sure what to talk about here. There aren’t any alignment rules, but who knows what the future will entail. Maybe 2 byte alignment for reasons]

## 3.5 	MEMORY ACCESSES

[Again not sure how to word or what to put here]

# CHAPTER 4
INSTRUCTION ENCODING

## 4.1 	BASE INSTRUCTION FORMAT

The base instruction format has a width of 2 bytes. The bits are arranged as follows:

| 15 ... 13 | 12 ... 9 | 8 ... 6 | 5 ... 3 | 2 ... 1 |
| --- | --- | --- | --- | --- |
| class | opcode | Destination operand | Source operand 1 | Source operand 2 |

## 4.2 	INSTRUCTION CLASSES

Instructions come in 1 of 4 classes: ALU operations, data movement, control flow and system instructions. [probs should be more here]

## 4.3 	OPCODE ENCODING

## 4.4 	PREFIX ENCODING

## 4.5		INSTRUCTION LENGTHS

# CHAPTER 5
ADDRESSING MODES

## 5.1 	REGISTER

## 5.2 	IMMEDIATE

## 5.3 	REGISTER INDIRECT

## 5.4 	ABSOLUTE

## 5.5 	REGISTER INDIRECT + DISPLACEMENT

## 5.6 	SIB

## 5.7 	SIB + DISPLACEMENT

# CHAPTER 6
OPERAND SIZES

## 6.1		DEFAULT OPERAND SIZE

Operand sizes by default are 8-bit when operands are immediates, absolutes, displacements and memory operations. Instructions only using registers default to native word size.

## 6.2		AEX PREFIX

AEX prefix is a byte starting with 0x9 and the lower nibble encodes instruction size.

| Encoding | Size |
| --- | --- |
| 0000 | 8-bit |
| 0001 | 16-bit |
| 0010-1111 | Reserved |

## 6.3 	IMMEDIATE SIZES

## 6.4 	MEMORY ACCESS SIZES

## 6.5		DISPLACEMENT SIZES

# CHAPTER 7
INSTRUCTION SET REFERENCE

## 7.1		ARITHMETIC INSTRUCTIONS

ADD
===

Operation
---------
dst = src1 + src2

Syntax
------
ADD Rd, Rs, Rt
ADD Rd, Rs, Imm8/Imm16

Operands
--------
Rd GP Destination register
Rs GP Source register
Rt GP Source register
Imm8 8-bit immediate value
Imm16 16-bit immediate value

Flags
-----
Z Updated
N Updated
C Updated
O Updated

Exceptions
----------
None

Encoding
--------
Opcode: 0x00

SUB
===

Operation
---------
dst = src1 - src2

Syntax
------
SUB Rd, Rs, Rt
SUB Rd, Rs, Imm8/Imm16

Operands
--------
Rd GP Destination register
Rs GP Source register
Rt GP Source register 
Imm8 8-bit immediate value
Imm16 16-bit immediate value

Flags
-----
Z Updated
N Updated
C Updated
O Updated

Exceptions
----------
None

Encoding
--------
Opcode: 0x02

CMP
===

Operation
---------
src1 - src2

Syntax
------
CMP Rs, Rt
CMP Rs, Imm8/Imm16

Operands
-------- 
Rs GP Source register
Rt GP Source register 
Imm8 8-bit immediate value
Imm16 16-bit immediate value

Flags
-----
Z Updated
N Updated
C Updated
O Updated

Exceptions
----------
None

Encoding
--------
Opcode: 0x0C

## 7.2 	LOGICAL INSTRUCTIONS

AND
===

Operation
---------
dst = src1 & src2

Syntax
------
AND Rd, Rs, Rt
AND Rd, Rs, Imm8/Imm16

Operands
--------
Rd GP Destination register
Rs GP Source register
Rt GP Source register 
Imm8 8-bit immediate value
Imm16 16-bit immediate value

Flags
-----
Z Updated
N Updated
C Set to 0
O Set to 0

Exceptions
----------
None

Encoding
--------
Opcode: 0x04

OR
===

Operation
---------
dst = src1 | src2

Syntax
------
OR Rd, Rs, Rt
OR Rd, Rs, Imm8/Imm16

Operands
--------
Rd GP Destination register
Rs GP Source register
Rt GP Source register 
Imm8 8-bit immediate value
Imm16 16-bit immediate value

Flags
-----
Z Updated
N Updated
C Set to 0
O Set to 0

Exceptions
----------
None

Encoding
--------
Opcode: 0x06

XOR
===

Operation
---------
dst = src1 ^ src2

Syntax
------
XOR Rd, Rs, Rt
XOR Rd, Rs, Imm8/Imm16

Operands
--------
Rd GP Destination register
Rs GP Source register
Rt GP Source register 
Imm8 8-bit immediate value
Imm16 16-bit immediate value

Flags
-----
Z Updated
N Updated
C Set to 0
O Set to 0

Exceptions
----------
None

Encoding
--------
Opcode: 0x08

NOT
===

Operation
---------
dst = !src1

Syntax
------
NOT Rd, Rs
NOT Rd, Imm8/Imm16

Operands
--------
Rd GP Destination register
Rs GP Source register 
Imm8 8-bit immediate value
Imm16 16-bit immediate value

Flags
-----
Z Updated
N Updated
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x0A

TEST
===

Operation
---------
src1 & src2

Syntax
------
TEST Rs, Rt
TEST Rs, Imm8/Imm16

Operands
-------- 
Rs GP Source register
Rt GP Source register 
Imm8 8-bit immediate value
Imm16 16-bit immediate value

Flags
-----
Z Updated
N Updated
C Set to 0
O Set to 0

Exceptions
----------
None

Encoding
--------
Opcode: 0x0E

## 7.3		DATA MOVEMENT INSTRUCTIONS

MOV
===

Operation
---------
dst = src1

Syntax
------
MOV Rd, Rs
MOV Rd, Imm8/Imm16

Operands
--------
Rd GP Destination register
Rs GP Source register 
Imm8 8-bit immediate value
Imm16 16-bit immediate value

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x20

MOVSR
===

Operation
---------
dst = src1

Syntax
------
MOVSR Rd, Rs
MOVSR Rd, Imm8/Imm16

Operands
--------
Rd GP Destination register
Rs SP Source register 
Imm8 8-bit immediate value
Imm16 16-bit immediate value

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x22

MOVRS
===

Operation
---------
dst = src1

Syntax
------
MOVRS Rd, Rs
MOVRS Rd, Imm8/Imm16

Operands
--------
Rd SP Destination register
Rs GP Source register
Imm8 8-bit immediate value
Imm16 16-bit immediate value

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x24

MOVS
===

Operation
---------
dst = src1

Syntax
------
MOVS Rd, Rs 

Operands
--------
Rd SP Destination register
Rs SP Source register 

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x2C

PUSH
===

Operation
---------
[SP] = src1

Syntax
------
PUSH Rs
MOV Imm8/Imm16

Operands
-------- 
Rs GP Source register 
Imm8 8-bit immediate value
Imm16 16-bit immediate value

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x26

POP
===

Operation
---------
dst = [SP]

Syntax
------
POP Rd 

Operands
-------- 
Rd GP Dest register 

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x28

PUSHS
===

Operation
---------
[SP] = src1

Syntax
------
PUSHS Rs 

Operands
-------- 
Rs SP Source register

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x2E

POPS
===

Operation
---------
src1 = [SP]

Syntax
------
POPS Rd 

Operands
-------- 
Rd SP Dest register 

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x30

LEA
===

Operation
---------
dst = memory operation address computation idk

Syntax
------
LEA Rd, insert stuff here 

Operands
-------- 
Rd GP Dest register 

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x2A

## 7.4 	CONTROL FLOW INSTRUCTIONS

Mnemonic
===

Operation
---------
[wololo]

Syntax
------
[wololo] 

Operands
-------- 
[wololo]

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x??

Mnemonic
===

Operation
---------
[wololo]

Syntax
------
[wololo] 

Operands
-------- 
[wololo]

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x??

Mnemonic
===

Operation
---------
[wololo]

Syntax
------
[wololo] 

Operands
-------- 
[wololo]

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x??

Mnemonic
===

Operation
---------
[wololo]

Syntax
------
[wololo] 

Operands
-------- 
[wololo]

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x??

Mnemonic
===

Operation
---------
[wololo]

Syntax
------
[wololo] 

Operands
-------- 
[wololo]

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x??

Mnemonic
===

Operation
---------
[wololo]

Syntax
------
[wololo] 

Operands
-------- 
[wololo]

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x??

Mnemonic
===

Operation
---------
[wololo]

Syntax
------
[wololo] 

Operands
-------- 
[wololo]

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x??

Mnemonic
===

Operation
---------
[wololo]

Syntax
------
[wololo] 

Operands
-------- 
[wololo]

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x??

Mnemonic
===

Operation
---------
[wololo]

Syntax
------
[wololo] 

Operands
-------- 
[wololo]

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x??

Mnemonic
===

Operation
---------
[wololo]

Syntax
------
[wololo] 

Operands
-------- 
[wololo]

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x??

Mnemonic
===

Operation
---------
[wololo]

Syntax
------
[wololo] 

Operands
-------- 
[wololo]

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x??

Mnemonic
===

Operation
---------
[wololo]

Syntax
------
[wololo] 

Operands
-------- 
[wololo]

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x??

Mnemonic
===

Operation
---------
[wololo]

Syntax
------
[wololo] 

Operands
-------- 
[wololo]

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x??

Mnemonic
===

Operation
---------
[wololo]

Syntax
------
[wololo] 

Operands
-------- 
[wololo]

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x??

Mnemonic
===

Operation
---------
[wololo]

Syntax
------
[wololo] 

Operands
-------- 
[wololo]

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x??

Mnemonic
===

Operation
---------
[wololo]

Syntax
------
[wololo] 

Operands
-------- 
[wololo]

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x??

Mnemonic
===

Operation
---------
[wololo]

Syntax
------
[wololo] 

Operands
-------- 
[wololo]

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x??

Mnemonic
===

Operation
---------
[wololo]

Syntax
------
[wololo] 

Operands
-------- 
[wololo]

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x??

## 7.5		SYSTEM INSTRUCTIONS

NOP
===

Operation
---------
Does nothing

Syntax
------
NOP 

Operands
-------- 
None

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x60

HLT
===

Operation
---------
Halts operation of the processor

Syntax
------
HLT

Operands
-------- 
None

Flags
-----
Z Untouched
N Untouched
C Untouched
V Untouched

Exceptions
----------
None

Encoding
--------
Opcode: 0x62

# CHAPTER 8
FLAGS AND CONDITION CODES

## 8.1		ZERO FLAG

## 8.2		CARRY FLAG

## 8.3 	OVERFLOW FLAG

## 8.4 	SIGN FLAG

## 8.5		CONDITION EVALUATION

# CHAPTER 9
PROGRAMMING EXAMPLES

## 9.1 FUNCTION CALL EXAMPLE

## 9.2 LOOP EXAMPLE

## 9.3 MEMORY COPY EXAMPLE

## 9.4 STACK USAGE EXAMPLE

# APPENDIX A
OPCODE SUMMARY

# APPENDIX B
PREFIX SUMMARY

# APPENDIX C
REGISTER SUMMARY

# APPENDIX D
INSTRUCTION QUICK REFERENCE